|  | ### S13.0 | ### IP Data Block Format |
| --- | --- | --- |
|  | S13.1 | SBC IP packets are always included in a consistent binary data block format. |
|  | S13.2 | A block of data shall is identified within the binary stream as follows… |
|  | S13.2.1 | - The first byte represents a Data Link Escape (DLE) with a hexadecimal value of 10h. |
|  | S13.2.2 | - The next byte represents a Start of Transmission (STX) with a value of 2h |
|  | S13.2.3 | - The next two bytes represents a Function Length indicating the total number of bytes that follow that make up the function’s data. |
|  | S13.2.4 | - Immediately following the function data is Data Link Escape (DLE) byte (10h). |
|  | S13.2.5 | - The final byte is the End of Transmission byte (ETX) with a value of 3h. |
|  | S13.2 | The software will acquire IP data packets when receiving FUN26 data. |
|  | ### S14.0 | ### Function 26 – Weld-Characteristic Data |
|  | S14.1 | FUN26 is included as an IP Data Packet (see S13.0) |
|  | S14.1.3 | Bold information in the table below represents the Function’s specific data the non-bold information is part of the standard IP Data Block Format (see S13.0) Name; Type; Example (HEX); USCS Conversion; Metric Conversion. DLE = 0x2; UINT8; 0x10; Data Link Escape; Data Link Escape. STX = 0x2; UINT8; 0x02; Start of Transmission; Start of Transmission. CNT = 118; UINT16; 0x00, 0x76; Length of Function; Length of Function. FUN = 26; UINT16; 0x00, 0x1A; Function Type; Function Type. Cycle Count; SINT32; 0x00,0x00,0x27,0x10; 10000 = 10000 cycles; 10000 = 10000 cycles. Hour; UINT16; 0x00,0x0C; 12 hour; 12 hour. Minute; UINT8; 0x3B; 59 = 59 minutes; 59 = 59 minutes. Second; UINT8; 0x3B; 59 = 59 seconds; 59 = 59 seconds. Year; UINT16; 0x07,0xD6; 2006 = 2006 year; 2006 = 2006 year. Month; UINT8; 0x0C; 12 = 12 month (December); 12 = 12 month (December). Day; UINT8; 0x1F; 31 = 31 day; 31 = 31 day. Actual Weld Time; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1.000 Seconds; 1000 = 1.000 Seconds. Total Energy; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 100.0 joules; 1000 = 100.0 joules. Actual Distance; SINT32; 0x00,0x00,0x27,0x10; 10000 = 1.0000 inch; 10000 = 25.4 mm. Actual Weld Collapse; SINT32; 0x00,0x00,0x27,0x10; 10000 = 1.0000 inch; 10000 = 25.4 mm. Actual Total Collapse; SINT32; 0x00,0x00,0x27,0x10; 10000 = 1.0000 inch; 10000 = 25.4 mm. Actual Start Frequency; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1000 Hz; 1000 = 1000 Hz. Actual End Frequency; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1000 Hz; 1000 = 1000 Hz. Frequency Change; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1000 Hz; 1000 = 1000 Hz. Frequency Minimum; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1000 Hz; 1000 = 1000 Hz. Frequency Maximum; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1000 Hz; 1000 = 1000 Hz. Actual Velocity; SINT32; 0x00,0x00,0x03,0xE8; 1000 = 100.0 in/sec; 1000 = 25.4 mm /sec. Trigger Distance; SINT32; 0x00,0x00,0x27,0x10; 10000 = 1.0000 inch; 10000 = 25.4 mm. Reject Part Alarms; UINT32; (See S18.0); (See S18.0). Suspect Part Alarms; UINT32; (See S19.0); (See S19.0). No Cycle Alarms; UINT32; (See S20.0); (See S20.0). Overload Alarms; UINT32; (See S21.0); (See S21.0). Equipment Failure Alarms; UINT32; (See S22.0); (See S22.0). Cycle Modified Alarms; UINT32; (See S23.0); (See S23.0). Preset Number; UINT8; from 0 to 16; from 0 to 16. Language; UINT8; (See S16.0); (See S16.0). Units; UINT8; (See S17.0); (See S17.0). Preset Limits Changed; UINT8; 0 = Not Changed 1 = Changed. Note Alarms; UINT32; (see S25.0); (see S25.0). Actual Amplitude Start; SINT32; 0x00,0x00,0x00,0x64; 100 = 100%; 100 = 100%. Actual Amplitude End; SINT32; 0x00,0x00,0x00,0x64; 100 = 100%; 100 = 100%. Weld Mode; UINT8; (See S26.0); (See S26.0). Power Supply Control Flags; UINT8; <undefined>; <undefined>. Peak Power; UINT16; 0x00,0x00,0x03,0xE8; 1000 = 100.0%; 1000 = 100.0%. Weld Pressure; UINT16; 0x00,0x00,0x03,0xE8; 1000 = 100.0 PSI; 1000 = 689kPa. Weld Force Maximum; UINT16; 0x00,0x00,0x03,0xE8; 1000 = 100.0 LB; 1000 = 444.8222 N (Newton). Hold Force Maximum; UINT16; 0x00,0x00,0x03,0xE8; 1000 = 100.0 LB; 1000 = 444.8222 N (Newton). Total Cycle Time (originally Actual Force Start (UINT16), Actual Force End (UINT16); SINT32; 0x00,0x00,0x03,0xE8; 1000 = 1.000 Seconds; 1000 = 1.000 Seconds. Trigger Force; UINT16; 0x00,0x00,0x03,0xE8; 1000 = 100.0 LB; 1000 = 444.8222 N. Checksum; UNIT16; Reserved Byte; Reserved Byte. DLE = 0x10; UINT8; 0x10; Data Link Escape; Data Link Escape. ETX = 0x2; UINT8; 0x02; End of Transmission; End of Transmission. |

|  | S16.0 | Language Constant Values: These values are used in FUN26. 0 = English 1= German 2 = Spanish 3 = Italian 4 = French 5 = Chinese 6 = Chinese Traditional 7 = Japanese 8 = Korean |
| --- | --- | --- |
|  | S17.0 | Unit Constant Values: These values are used in FUN26. 0 = USCS 1 = Metric 2 = Metric2 |
|  | S18.0 | Function 26: Reject Part Alarm field have these bit values Bit Assignment; Alarm Text. Bit01; -R Energy Limit. Bit02; +R Energy Limit. Bit03; -R Pk Power Limit. Bit04; +R Pk Power Limit. Bit05; -R Col Dist Limit. Bit06; +R Col Dist Limit. Bit07; -R Abs Dist Limit. Bit08; +R Abs Dist Limit. Bit09; -R Trig Dist Limit. Bit10; +R Trig Dist Limit. Bit11; -R Weld Force Limit. Bit12; +R Weld Force Limit. Bit13; -R Time Limit. Bit14; +R Time Limit. Bit15-17; <undefined>. Bit18; LL Not Reached. Bit19-32; <undefined>. |
|  | S19.0 | Function 26: Suspect Part Alarm have these bit values Bit Assignment; Alarm Text. Bit01; -S Energy Limit. Bit02; +S Energy Limit. Bit03; -S Pk Power Limit. Bit04; +S Pk Power Limit. Bit05; -S Col Dist Limit. Bit06; +S Col Dist Limit. Bit07; -S Abs Dist Limit. Bit08; +S Abs Dist Limit. Bit09; -S Trig Dist Limit. Bit10; +S Trig Dist Limit. Bit11; -S Weld Force Limit. Bit12; +S Weld Force Limit. Bit13; -S Time Limit. Bit14; +S Time Limit. Bit15-32; <undefined>. |
|  | S20.0 | Function 26: No Cycle Alarm has these bit values. Bit Assignment; Alarm Text. Bit01; Upper Limit Timeout. Bit02; <undefined>. Bit03; Trigger Before Pretrigger. Bit04; Trigger Timeout. Bit05; <undefined>. Bit06; LLS abort before TRS. Bit07; External Cycle Abort. Bit08; Missing Part Abort. Bit09; Abs Before Trigger. Bit10; Amp Step before Trigger. Bit11; F Step before Trigger. Bit12; Ground Detecte Cutoff. Bit13-32; <undefined>. |
|  | S21.0 | Function 26: Overload Alarm has these bit values. Bit Assignment; Alarm Text. Bit01; Test Overload. Bit02; Pretrigger Overload. Bit03; Seek Overload. Bit04; Power Supply Overload. Bit05; Cont Power Limit. Bit06; Afterburst Overload. Bit07; Pre-Weld Seek Overload. Bit08; Post Weld Seek Overload. Bit09-32; <undefined>. |
|  | S22.0 | Function 26: Equipment Failure Alarm has these bit values. Bit Assignment; Alarm Text. Bit01; Encoder Failed. Bit02; Upper Limit Switch. Bit03; Upper Limit Switch. Bit04; Door Sw Fail. Bit05; <undefined>. Bit06; Solenoid Drive Fail. Bit07; Thermal Overload. Bit08; Preset Data/BBR. Bit09; Horn Return Timeout. Bit10; Actuator NovRam. Bi11; P/S NovRam. Bit12; Start Sw Time. Bit13; MPS Switch Failed. Bit14; Wrong Actuator. Bit15; Ultrasonic P/S. Bit16; Printer Buffer Full. Bit17; Start Switch Closed. Bit18; Pretrigger Timeout. Bit19; <undefined>. Bit20; Recalibrate Actuator. Bit21; Act Clear Function. Bit22; Stack. Bit23; Start Switches Lost. Bit24; Actuator Type. Bit25; Sys. Pres. Incorrect. Bit26-32; <undefined>. |
|  | S23.0 | Function 26: Cycle Modified Alarm has these bit values. Bit Assignment; Alarm Text. Bit01; Trigger Lost in Hold. Bit02; Ground Detect Cutoff. Bit03; Maximum Timeout. Bit04; No Amplitude Step. Bit05; No Force Step. Bit06; No Amplitude Step. Bit07; No Force Step. Bit08; No Amplitude Step. Bit09; No Amplitude Step. Bit10; No Force Step. Bi11; No Amplitude Step. Bit12; No Amplitude Step. Bit13; No Force Step. Bit14; Trigger Lost in Weld. Bit15; External Cycle Abort. Bit16; Amp B Not Reached. Bit17; Amp Not Reached. Bit18; Amp A Not Reached. Bit19; Amp B Not Reached. Bit20; Amp Exceeded. Bit21; Energy Not Reached. Bit22; Trigger > End Force. Bit23; No Force Step. Bit24-32; <undefined>. |
|  | S24.0 | Function 26: Calibrate Alarms has these bit values. Bit Assignment; Alarm Text. Bit01-32; <undefined>. |
|  | S25.0 | Function 26: Note Alarms has these bit values. Bit Assignment; Alarm Text. Bit01; Act Clr Not Reached. Bit02; Max Energy Reached. Bit03; Printer Buffer 80%. Bit04; Cont Power Limit. Bit05; Peak Power Cutoff. Bit06; Absolute Cutoff. Bit07; Time Extended. Bit08; Act Recal Suggested. Bit09; Collapse Cutoff. Bit10; Act Clr Not Reached. Bit11-32; <undefined>. |
|  | S26.0 | Function 26: Weld Mode has these bit values. Bit Assignment; Mode Text. Bit01; Time. Bit02; Energy. Bit03; Peak Power. Bit04; Collapse. Bit05; Absolute. Bit06; Gnd Detect. |
|  | S27.0 | Control Level (UINT16) has these constant values. Name; Value (Hex). CL_LEVEL_DEPOT; 01. CL_LEVEL_T; 10. CL_LEVEL_E; 20. CL_LEVEL_A; 30. CL_LEVEL_EA; 40. CL_LEVEL_D; 50. CL_LEVEL_F; 60. CL_LEVEL_TEST; FFFF. |
|  | S28.0 | Actuator (UINT16) has these constant values. Name; Value (Hex). ACTUATOR_AE; 2. ACTUATOR_AO; 4. ACTUATOR_AED; 8. ACTUATOR_AOD; 10. ACTUATOR_AES; 20. ACTUATOR_AOS; 40. ACTUATOR_AEF; 80. ACTUATOR_AOF; 100. ACTUATOR_HGUN; 200. ACTUATOR_DONTCARE; FFFF. |